# CPU project

eProjectCPU

Designing and Building a Processor Using SystemVerilog and an FPGA

1. Contents

2. Introduction

2.1 Why I Started This Project

2.2 The Purpose of This Book

2.3 Who This Book Is For

2.4 What You Will Build

2.5 What You Will Learn

2.6 Final Project Outcome

2.7 Project Requirements

2.8 Recommended Previous Knowledge

2.9 Skills Developed During the Project

2.9.1 SystemVerilog Programming

2.9.2 Digital Logic Design

2.9.3 Computer Architecture

2.9.4 FPGA Development

2.9.5 Testbench Development

2.9.6 Processor Verification

2.9.7 Timing Analysis

2.9.8 Git and GitHub

2.9.9 Engineering Documentation

2.9.10 Technical Problem-Solving

2.10 Overview of the Project Stages

3. Hardware

3.1 FPGA Development Board

3.1.1 What Is an FPGA?

3.1.2 How Does an FPGA Work?

3.1.3 FPGA Versus a Microcontroller

3.1.4 FPGA Versus a Raspberry Pi

3.1.5 Main Resources Inside an FPGA

3.1.6 Lookup Tables

3.1.7 Flip-Flops

3.1.8 Block RAM

3.1.9 DSP Slices

3.1.10 Clock Management Resources

3.1.11 Input and Output Pins

3.1.12 Specifications of the FPGA Board

3.1.13 FPGA Board Pinout

3.1.14 How the CPU Uses the FPGA

3.2 Computer

3.2.1 Recommended Computer Specifications

3.2.2 Supported Operating Systems

3.2.3 Required Storage Space

3.3 USB Programming Cable

3.3.1 Powering the FPGA Board

3.3.2 Programming the FPGA

3.3.3 JTAG Communication

3.4 Clock Source

3.4.1 What Is a Clock Signal?

3.4.2 Clock Frequency

3.4.3 Clock Period

3.4.4 Why the CPU Requires a Clock

3.4.5 Maximum Clock Frequency

3.5 Push Buttons

3.5.1 Reset Button

3.5.2 Manual Clock Button

3.6 Switches

3.6.1 Using Switches as CPU Inputs

3.6.2 Selecting Test Modes

3.7 LEDs

3.7.1 Displaying CPU Results

3.7.2 Displaying Processor Status

3.8 Seven-Segment Display

3.8.1 Displaying Register Values

3.8.2 Displaying Program Results

3.9 UART Interface

3.9.1 What Is UART?

3.9.2 Transmitting CPU Information

3.9.3 Viewing Results on a Computer

3.10 Hardware Handling and Safety

3.10.1 Electrostatic Discharge

3.10.2 Correct Board Voltage

3.10.3 Preventing Short Circuits

3.10.4 Connecting and Disconnecting the Board Safely

4. Software

4.1 Vivado

4.1.1 What Is Vivado?

4.1.2 Simulation

4.1.3 Synthesis

4.1.4 Implementation

4.1.5 Timing Analysis

4.1.6 Bitstream Generation

4.1.7 Hardware Manager

4.1.8 How to Download Vivado

4.1.9 How to Install Vivado

4.1.10 How to Select the Correct FPGA Devices

4.1.11 How to Create a Vivado Project

4.1.12 How to Select the Correct Board or FPGA Part

4.1.13 How to Add Design Sources

4.1.14 How to Add Simulation Sources

4.1.15 How to Add Constraints

4.1.16 How to Run a Simulation

4.1.17 How to Use the Waveform Viewer

4.1.18 How to Run Synthesis

4.1.19 How to Run Implementation

4.1.20 How to Generate a Bitstream

4.1.21 How to Program the FPGA

4.2 Visual Studio Code

4.2.1 What Is Visual Studio Code?

4.2.2 How to Install Visual Studio Code

4.2.3 How to Install SystemVerilog Extensions

4.2.4 How to Open the Project Folder

4.2.5 Editing SystemVerilog Files

4.3 Git

4.3.1 What Is Git?

4.3.2 Why Use Version Control?

4.3.3 How to Install Git

4.3.4 How to Create a Git Repository

4.3.5 Git Status

4.3.6 Adding Files

4.3.7 Creating Commits

4.3.8 Viewing Commit History

4.3.9 Creating Branches

4.3.10 Using a .gitignore File

4.4 GitHub

4.4.1 What Is GitHub?

4.4.2 How to Create a GitHub Repository

4.4.3 How to Connect a Local Repository

4.4.4 How to Push Files to GitHub

4.4.5 How to Pull Changes

4.4.6 Writing a README

4.4.7 Using GitHub Issues

4.4.8 Creating Releases

4.5 Codex and AI-Assisted Development

4.5.1 What Is Codex?

4.5.2 Installing and Setting Up Codex

4.5.3 Using Codex Inside the Project Repository

4.5.4 Asking Codex to Explain Code

4.5.5 Asking Codex to Create Testbenches

4.5.6 Asking Codex to Review RTL

4.5.7 Asking Codex to Help Debug Errors

4.5.8 Reviewing AI-Generated Code

4.5.9 Limitations of AI-Assisted Development

5. Module 1: Setting Up the FPGA CPU Project

5.1 Learning Objectives

5.2 Build Map

5.3 Background Theory

5.3.1 What Is Register Transfer Level Design?

5.3.2 What Is Simulation?

5.3.3 What Is Synthesis?

5.3.4 What Is Implementation?

5.3.5 What Is a Testbench?

5.3.6 Simulation Versus FPGA Hardware

5.4 Tools and Software Required

5.5 Step 1: Create the Project Folder

5.6 Step 2: Create the Project Folder Structure

5.7 Step 3: Create the Git Repository

5.8 Step 4: Create the GitHub Repository

5.9 Step 5: Create the Vivado Project

5.10 Step 6: Add the First SystemVerilog Module

5.11 Step 7: Create the First Testbench

5.12 Step 8: Run the First Simulation

5.13 Step 9: Commit and Push the Project

5.14 Testing Your Project Setup

5.15 Troubleshooting

5.16 Good Practice Notes

5.17 Summary

5.18 What You Should Have Now

5.19 What to Expect in Module 2

6. Module 2: Learning SystemVerilog Through Simple Circuits

6.1 Learning Objectives

6.2 Build Map

6.3 Background Theory

6.3.1 SystemVerilog Modules

6.3.2 Inputs and Outputs

6.3.3 The Logic Data Type

6.3.4 Continuous Assignments

6.3.5 The always_comb Block

6.3.6 The always_ff Block

6.3.7 Blocking Assignments

6.3.8 Non-Blocking Assignments

6.3.9 Parameters

6.3.10 Clock Signals

6.3.11 Reset Signals

6.4 Tools and Files Required

6.5 Step 1: Create an AND Gate

6.6 Step 2: Create a Multiplexer

6.7 Step 3: Create a D-Type Register

6.8 Step 4: Create a Counter

6.9 Step 5: Create Testbenches

6.10 Step 6: Read the Simulation Waveforms

6.11 Testing

6.12 Troubleshooting

6.13 Common SystemVerilog Mistakes

6.14 Summary

6.15 What You Should Have Now

6.16 What to Expect in Module 3

7. Module 3: Designing the CPU Instruction Set

7.1 Learning Objectives

7.2 Build Map

7.3 Background Theory

7.3.1 What Is a Processor?

7.3.2 What Is an Instruction?

7.3.3 What Is an Instruction Set Architecture?

7.3.4 What Is an Opcode?

7.3.5 Source and Destination Operands

7.3.6 CPU Registers

7.3.7 Immediate Values

7.3.8 Memory Addresses

7.3.9 Branches and Jumps

7.4 Tools and Files Required

7.5 Step 1: Select the Processor Data Width

7.6 Step 2: Select the Instruction Width

7.7 Step 3: Select the Number of Registers

7.8 Step 4: Define the Instruction Formats

7.9 Step 5: Define the Opcodes

7.10 Step 6: Define Arithmetic Instructions

7.11 Step 7: Define Logical Instructions

7.12 Step 8: Define Immediate Instructions

7.13 Step 9: Define Load and Store Instructions

7.14 Step 10: Define Branch Instructions

7.15 Step 11: Define Jump Instructions

7.16 Step 12: Define the HALT Instruction

7.17 Step 13: Create the Complete Instruction Table

7.18 Step 14: Create Instruction Encoding Diagrams

7.19 Step 15: Manually Encode a Test Program

7.20 Testing the Instruction Set

7.21 Troubleshooting

7.22 Summary

7.23 What You Should Have Now

7.24 What to Expect in Module 4

8. Module 4: Building the Arithmetic Logic Unit

8.1 Learning Objectives

8.2 Build Map

8.3 Background Theory

8.3.1 What Is an Arithmetic Logic Unit?

8.3.2 Binary Addition

8.3.3 Binary Subtraction

8.3.4 Bitwise AND

8.3.5 Bitwise OR

8.3.6 Bitwise XOR

8.3.7 Logical Shifts

8.3.8 Signed and Unsigned Numbers

8.3.9 Carry

8.3.10 Overflow

8.3.11 Zero Flag

8.3.12 Negative Flag

8.4 Tools and Files Required

8.5 Step 1: Define the ALU Interface

8.6 Step 2: Define the ALU Operation Codes

8.7 Step 3: Implement Addition

8.8 Step 4: Implement Subtraction

8.9 Step 5: Implement Logical Operations

8.10 Step 6: Implement Shift Operations

8.11 Step 7: Implement Comparison Operations

8.12 Step 8: Add Status Flags

8.13 Step 9: Add a Default Operation

8.14 Step 10: Create the ALU Testbench

8.15 Step 11: Create Self-Checking Tests

8.16 Testing the ALU

8.17 Reading the ALU Waveform

8.18 Troubleshooting

8.19 Summary

8.20 What You Should Have Now

8.21 What to Expect in Module 5

9. Module 5: Building the Register File

9.1 Learning Objectives

9.2 Build Map

9.3 Background Theory

9.3.1 What Is a CPU Register?

9.3.2 Why CPUs Use Registers

9.3.3 What Is a Register File?

9.3.4 Read Ports

9.3.5 Write Ports

9.3.6 Synchronous Writes

9.3.7 Combinational Reads

9.3.8 The Zero Register

9.4 Tools and Files Required

9.5 Step 1: Define the Register File Interface

9.6 Step 2: Create the Register Array

9.7 Step 3: Add the First Read Port

9.8 Step 4: Add the Second Read Port

9.9 Step 5: Add the Write Port

9.10 Step 6: Add the Write Enable

9.11 Step 7: Add Reset Behaviour

9.12 Step 8: Protect the Zero Register

9.13 Step 9: Create the Register File Testbench

9.14 Testing the Register File

9.15 Reading the Register File Waveform

9.16 Troubleshooting

9.17 Summary

9.18 What You Should Have Now

9.19 What to Expect in Module 6

10. Module 6: Building the Program Counter and Instruction Memory

10.1 Learning Objectives

10.2 Build Map

10.3 Background Theory

10.3.1 What Is a Program Counter?

10.3.2 What Is Instruction Memory?

10.3.3 The Instruction Fetch Process

10.3.4 Sequential Program Execution

10.3.5 Program Counter Reset

10.3.6 Program Counter Increment

10.3.7 Program Memory Files

10.4 Tools and Files Required

10.5 Step 1: Define the Program Counter Interface

10.6 Step 2: Create the Program Counter Register

10.7 Step 3: Add Reset Behaviour

10.8 Step 4: Add the Program Counter Increment

10.9 Step 5: Create Instruction Memory

10.10 Step 6: Create a Program Memory File

10.11 Step 7: Load the Program Using readmemh

10.12 Step 8: Connect the Program Counter to Instruction Memory

10.13 Step 9: Create the Fetch Testbench

10.14 Testing Instruction Fetch

10.15 Reading the Fetch Waveform

10.16 Troubleshooting

10.17 Summary

10.18 What You Should Have Now

10.19 What to Expect in Module 7

11. Module 7: Building Data Memory and the Load/Store System

11.1 Learning Objectives

11.2 Build Map

11.3 Background Theory

11.3.1 What Is Data Memory?

11.3.2 Instruction Memory Versus Data Memory

11.3.3 Memory Addresses

11.3.4 Memory Reads

11.3.5 Memory Writes

11.3.6 Memory Write Enable

11.3.7 Synchronous Memory

11.3.8 FPGA Block RAM

11.4 Tools and Files Required

11.5 Step 1: Define the Data Memory Interface

11.6 Step 2: Create the Memory Array

11.7 Step 3: Add Memory Reads

11.8 Step 4: Add Memory Writes

11.9 Step 5: Add the Write Enable

11.10 Step 6: Add Memory Initialisation

11.11 Step 7: Create the Data Memory Testbench

11.12 Testing Data Memory

11.13 Reading the Memory Waveform

11.14 Troubleshooting

11.15 Summary

11.16 What You Should Have Now

11.17 What to Expect in Module 8

12. Module 8: Designing the Control Unit

12.1 Learning Objectives

12.2 Build Map

12.3 Background Theory

12.3.1 What Is a Control Unit?

12.3.2 Datapath Versus Control Path

12.3.3 Instruction Decoding

12.3.4 ALU Control

12.3.5 Register Write Control

12.3.6 Memory Write Control

12.3.7 Operand Selection

12.3.8 Write-Back Selection

12.3.9 Branch Control

12.3.10 Jump Control

12.3.11 Illegal Instructions

12.4 Tools and Files Required

12.5 Step 1: Define the Control Unit Interface

12.6 Step 2: Create the Control Signal Table

12.7 Step 3: Decode Arithmetic Instructions

12.8 Step 4: Decode Logical Instructions

12.9 Step 5: Decode Immediate Instructions

12.10 Step 6: Decode Load Instructions

12.11 Step 7: Decode Store Instructions

12.12 Step 8: Decode Branch Instructions

12.13 Step 9: Decode Jump Instructions

12.14 Step 10: Decode the HALT Instruction

12.15 Step 11: Add Default Control Values

12.16 Step 12: Create the Control Unit Testbench

12.17 Testing the Control Unit

12.18 Reading the Control Waveform

12.19 Troubleshooting

12.20 Summary

12.21 What You Should Have Now

12.22 What to Expect in Module 9

13. Module 9: Connecting the Complete Single-Cycle Processor

13.1 Learning Objectives

13.2 Build Map

13.3 Background Theory

13.3.1 What Is a Datapath?

13.3.2 The Fetch Stage

13.3.3 The Decode Stage

13.3.4 The Execute Stage

13.3.5 The Memory Stage

13.3.6 The Write-Back Stage

13.3.7 Single-Cycle Execution

13.3.8 The Critical Path

13.4 Tools and Files Required

13.5 Complete Processor Block Diagram

13.6 Step 1: Create the CPU Top-Level Module

13.7 Step 2: Connect the Program Counter

13.8 Step 3: Connect Instruction Memory

13.9 Step 4: Extract the Instruction Fields

13.10 Step 5: Connect the Control Unit

13.11 Step 6: Connect the Register File

13.12 Step 7: Connect the ALU

13.13 Step 8: Connect Data Memory

13.14 Step 9: Add Operand Multiplexers

13.15 Step 10: Add the Write-Back Multiplexer

13.16 Step 11: Add Branch Logic

13.17 Step 12: Add Jump Logic

13.18 Step 13: Add the HALT Function

13.19 Step 14: Create the CPU Testbench

13.20 Step 15: Create the First CPU Program

13.21 Step 16: Run the First Complete Program

13.22 Testing the Complete CPU

13.23 Reading the Complete CPU Waveform

13.24 Troubleshooting

13.25 Summary

13.26 What You Should Have Now

13.27 What to Expect in Module 10

14. Module 10: Testing and Debugging the Complete Processor

14.1 Learning Objectives

14.2 Build Map

14.3 Background Theory

14.3.1 Why Processor Verification Is Important

14.3.2 Unit Testing

14.3.3 Integration Testing

14.3.4 Regression Testing

14.3.5 Self-Checking Testbenches

14.3.6 Assertions

14.3.7 Expected Results

14.3.8 Test Coverage

14.4 Tools and Files Required

14.5 Step 1: Create Arithmetic Test Programs

14.6 Step 2: Create Logical Test Programs

14.7 Step 3: Create Immediate Instruction Tests

14.8 Step 4: Create Load and Store Tests

14.9 Step 5: Create Branch Tests

14.10 Step 6: Create Jump Tests

14.11 Step 7: Create Loop Tests

14.12 Step 8: Create a Fibonacci Program

14.13 Step 9: Create a Memory Copy Program

14.14 Step 10: Test the HALT Instruction

14.15 Step 11: Test Illegal Instructions

14.16 Step 12: Add Assertions

14.17 Step 13: Create the Regression Test Suite

14.18 Recommended CPU Debugging Order

14.18.1 Check the Clock

14.18.2 Check the Reset

14.18.3 Check the Program Counter

14.18.4 Check the Instruction

14.18.5 Check the Opcode

14.18.6 Check the Control Signals

14.18.7 Check the Register Values

14.18.8 Check the ALU Inputs

14.18.9 Check the ALU Result

14.18.10 Check the Memory Signals

14.18.11 Check the Write-Back Data

14.19 Troubleshooting

14.20 Summary

14.21 What You Should Have Now

14.22 What to Expect in Module 11

15. Module 11: Synthesising the Processor in Vivado

15.1 Learning Objectives

15.2 Build Map

15.3 Background Theory

15.3.1 What Is Synthesis?

15.3.2 What Is Technology Mapping?

15.3.3 What Are Lookup Tables?

15.3.4 What Are Flip-Flops?

15.3.5 What Is Block RAM?

15.3.6 What Are DSP Slices?

15.3.7 What Is an Inferred Latch?

15.3.8 What Is Resource Utilisation?

15.3.9 What Is a Critical Path?

15.4 Tools and Files Required

15.5 Step 1: Set the CPU as the Top Module

15.6 Step 2: Check the Sources

15.7 Step 3: Run RTL Analysis

15.8 Step 4: Open the Elaborated Design

15.9 Step 5: Run Synthesis

15.10 Step 6: Review Synthesis Warnings

15.11 Step 7: Open the Synthesis Schematic

15.12 Step 8: Read the Utilisation Report

15.13 Step 9: Record FPGA Resource Usage

15.14 Step 10: Run Implementation

15.15 Step 11: Review the Implementation Report

15.16 Testing the Synthesised Design

15.17 Troubleshooting

15.18 Summary

15.19 What You Should Have Now

15.20 What to Expect in Module 12

16. Module 12: Timing Analysis and Timing Closure

16.1 Learning Objectives

16.2 Build Map

16.3 Background Theory

16.3.1 What Is Timing Analysis?

16.3.2 Setup Time

16.3.3 Hold Time

16.3.4 Clock Period

16.3.5 Clock Frequency

16.3.6 Data Arrival Time

16.3.7 Data Required Time

16.3.8 Timing Slack

16.3.9 Positive Slack

16.3.10 Negative Slack

16.3.11 Combinational Logic Delay

16.3.12 Routing Delay

16.3.13 Logic Depth

16.3.14 Fan-Out

16.4 Tools and Files Required

16.5 Step 1: Create the Clock Constraint

16.6 Step 2: Run Timing Analysis

16.7 Step 3: Read the Timing Summary

16.8 Step 4: Find the Critical Path

16.9 Step 5: Identify the Slowest Logic

16.10 Step 6: Calculate the Maximum Clock Frequency

16.11 Step 7: Reduce Logic Depth

16.12 Step 8: Reduce Fan-Out

16.13 Step 9: Add Intermediate Registers Where Required

16.14 Step 10: Re-run Implementation

16.15 Step 11: Compare Timing Results

16.16 Testing Timing Improvements

16.17 Troubleshooting

16.18 Summary

16.19 What You Should Have Now

16.20 What to Expect in Module 13

17. Module 13: Programming the Processor onto the FPGA

17.1 Learning Objectives

17.2 Build Map

17.3 Background Theory

17.3.1 What Is a Bitstream?

17.3.2 What Is a Constraints File?

17.3.3 FPGA Pin Assignments

17.3.4 Memory-Mapped Input and Output

17.3.5 Hardware Reset

17.3.6 Clock Dividers

17.4 Tools and Hardware Required

17.5 Step 1: Create the FPGA Top-Level Module

17.6 Step 2: Connect the CPU Clock

17.7 Step 3: Connect the Reset Button

17.8 Step 4: Connect CPU Outputs to LEDs

17.9 Step 5: Add Memory-Mapped Output

17.10 Step 6: Add the Pin Constraints

17.11 Step 7: Run Synthesis and Implementation

17.12 Step 8: Generate the Bitstream

17.13 Step 9: Connect the FPGA Board

17.14 Step 10: Open Hardware Manager

17.15 Step 11: Program the FPGA

17.16 Step 12: Run the Hardware Test Program

17.17 Step 13: Compare Hardware and Simulation Results

17.18 Testing the FPGA Demonstration

17.19 Troubleshooting

17.20 Hardware Safety Notes

17.21 Summary

17.22 What You Should Have Now

17.23 What to Expect in Module 14

18. Module 14: Measuring CPU Performance

18.1 Learning Objectives

18.2 Build Map

18.3 Background Theory

18.3.1 What Is Processor Performance?

18.3.2 Clock Frequency

18.3.3 Clock Period

18.3.4 Instruction Count

18.3.5 Cycles per Instruction

18.3.6 Instructions per Cycle

18.3.7 Execution Time

18.3.8 Throughput

18.3.9 Latency

18.3.10 MIPS

18.3.11 Resource Utilisation

18.3.12 Power Consumption

18.3.13 Performance per LUT

18.3.14 Performance per Watt

18.4 Tools and Files Required

18.5 Step 1: Select a Benchmark Program

18.6 Step 2: Count the Program Instructions

18.7 Step 3: Count the Processor Cycles

18.8 Step 4: Measure the Clock Frequency

18.9 Step 5: Calculate Cycles per Instruction

18.10 Step 6: Calculate Execution Time

18.11 Step 7: Calculate MIPS

18.12 Step 8: Record Resource Utilisation

18.13 Step 9: Record Estimated Power

18.14 Step 10: Create a Performance Results Table

18.15 Comparing the CPU with the Arduino Uno

18.16 Comparing the CPU with a Raspberry Pi

18.17 Limitations of CPU Comparisons

18.18 Troubleshooting Performance Measurements

18.19 Summary

18.20 What You Should Have Now

18.21 What to Expect in Module 15

19. Module 15: Converting the Processor to a Five-Stage Pipeline

19.1 Learning Objectives

19.2 Build Map

19.3 Background Theory

19.3.1 Why Use a Pipeline?

19.3.2 Instruction-Level Parallelism

19.3.3 Pipeline Latency

19.3.4 Pipeline Throughput

19.3.5 Ideal Pipeline Performance

19.3.6 The Instruction Fetch Stage

19.3.7 The Instruction Decode Stage

19.3.8 The Execute Stage

19.3.9 The Memory Stage

19.3.10 The Write-Back Stage

19.3.11 Pipeline Registers

19.4 Tools and Files Required

19.5 Step 1: Divide the Datapath into Five Stages

19.6 Step 2: Add the IF/ID Pipeline Register

19.7 Step 3: Add the ID/EX Pipeline Register

19.8 Step 4: Add the EX/MEM Pipeline Register

19.9 Step 5: Add the MEM/WB Pipeline Register

19.10 Step 6: Pipeline the Control Signals

19.11 Step 7: Pipeline Register Addresses

19.12 Step 8: Pipeline Immediate Values

19.13 Step 9: Pipeline ALU Results

19.14 Step 10: Pipeline Memory Data

19.15 Step 11: Connect the Write-Back Stage

19.16 Step 12: Create the Pipelined CPU Testbench

19.17 Step 13: Test Independent Instructions

19.18 Reading the Pipeline Waveform

19.19 Troubleshooting

19.20 Summary

19.21 What You Should Have Now

19.22 What to Expect in Module 16

20. Module 16: Detecting and Solving Pipeline Hazards

20.1 Learning Objectives

20.2 Build Map

20.3 Background Theory

20.3.1 What Is a Pipeline Hazard?

20.3.2 Structural Hazards

20.3.3 Data Hazards

20.3.4 Control Hazards

20.3.5 Read After Write Hazards

20.3.6 Load-Use Hazards

20.3.7 Forwarding

20.3.8 Pipeline Stalls

20.3.9 Pipeline Bubbles

20.3.10 Pipeline Flushing

20.3.11 Branch Penalties

20.4 Tools and Files Required

20.5 Step 1: Create a Data Hazard Test

20.6 Step 2: Design the Forwarding Unit

20.7 Step 3: Add EX-to-EX Forwarding

20.8 Step 4: Add MEM-to-EX Forwarding

20.9 Step 5: Test the Forwarding Paths

20.10 Step 6: Design the Hazard Detection Unit

20.11 Step 7: Detect Load-Use Hazards

20.12 Step 8: Stall the Program Counter

20.13 Step 9: Stall the IF/ID Register

20.14 Step 10: Insert a Pipeline Bubble

20.15 Step 11: Detect Taken Branches

20.16 Step 12: Flush Incorrect Instructions

20.17 Step 13: Test Branch Hazards

20.18 Step 14: Run the Full Regression Suite

20.19 Testing the Hazard System

20.20 Reading Hazard Waveforms

20.21 Troubleshooting

20.22 Summary

20.23 What You Should Have Now

20.24 What to Expect in Module 17

21. Module 17: Evaluating the Pipelined Processor

21.1 Learning Objectives

21.2 Build Map

21.3 Background Theory

21.3.1 Pipeline Fill Time

21.3.2 Pipeline Drain Time

21.3.3 Ideal CPI

21.3.4 Actual CPI

21.3.5 Stall Cycles

21.3.6 Branch Penalties

21.3.7 Effective MIPS

21.4 Tools and Files Required

21.5 Step 1: Select the Benchmark Programs

21.6 Step 2: Measure the Pipelined Clock Frequency

21.7 Step 3: Count Completed Instructions

21.8 Step 4: Count Total Processor Cycles

21.9 Step 5: Calculate Average CPI

21.10 Step 6: Calculate Effective MIPS

21.11 Step 7: Measure Stall Cycles

21.12 Step 8: Measure Branch Penalties

21.13 Step 9: Record Resource Utilisation

21.14 Step 10: Record Estimated Power

21.15 Step 11: Record the 71.1 MIPS Result

21.16 Step 12: Explain the Measurement Conditions

21.17 Testing the Performance Results

21.18 Troubleshooting

21.19 Summary

21.20 What You Should Have Now

21.21 What to Expect in Module 18

22. Module 18: Comparing the Processor Versions

22.1 Learning Objectives

22.2 Build Map

22.3 The Single-Cycle Processor

22.4 The Multi-Cycle Processor

22.5 The Five-Stage Pipelined Processor

22.6 Clock Frequency Comparison

22.7 CPI Comparison

22.8 MIPS Comparison

22.9 Execution Time Comparison

22.10 LUT Comparison

22.11 Flip-Flop Comparison

22.12 Block RAM Comparison

22.13 DSP Comparison

22.14 Power Comparison

22.15 Performance per LUT Comparison

22.16 Performance per Watt Comparison

22.17 Critical Path Comparison

22.18 Area Versus Speed Trade-Off

22.19 Advantages of the Single-Cycle Processor

22.20 Disadvantages of the Single-Cycle Processor

22.21 Advantages of the Multi-Cycle Processor

22.22 Disadvantages of the Multi-Cycle Processor

22.23 Advantages of the Pipelined Processor

22.24 Disadvantages of the Pipelined Processor

22.25 Limitations of the Comparison

22.26 Summary

22.27 What You Should Have Now

22.28 What to Expect in Module 19

23. Module 19: Turning the Summer Project into a Final-Year Project

23.1 Learning Objectives

23.2 Build Map

23.3 Selecting a Research Question

23.4 Defining a Hypothesis

23.5 Defining Independent Variables

23.6 Defining Dependent Variables

23.7 Designing a Fair Experiment

23.8 Recording Repeatable Results

23.9 Analysing Results

23.10 Discussing Limitations

23.11 Possible Processor Upgrades

23.11.1 Branch Prediction

23.11.2 Static Branch Prediction

23.11.3 Dynamic Branch Prediction

23.11.4 Instruction Cache

23.11.5 Data Cache

23.11.6 Hardware Multiplication

23.11.7 Hardware Division

23.11.8 Interrupt Support

23.11.9 UART Peripheral

23.11.10 SPI Peripheral

23.11.11 GPIO Peripheral

23.11.12 Timer Peripheral

23.11.13 Memory-Mapped Input and Output

23.11.14 RISC-V Compatibility

23.11.15 Configurable Data Width

23.11.16 Configurable Register Count

23.11.17 Configurable Pipeline Depth

23.11.18 Formal Verification

23.11.19 Fault Injection

23.11.20 Power Optimisation

23.11.21 Multicore Processing

23.12 Example Final-Year Research Questions

23.13 Creating an Experimental Plan

23.14 Results to Collect

23.15 Risk Assessment

23.16 Presenting the Idea to a Supervisor

23.17 Final-Year Project Report Structure

23.18 Summary

24. Module 20: Documenting and Presenting the Project

24.1 Learning Objectives

24.2 Build Map

24.3 Writing the Project README

24.4 Creating Architecture Diagrams

24.5 Documenting the Instruction Set

24.6 Documenting SystemVerilog Modules

24.7 Documenting Testbenches

24.8 Recording Simulation Results

24.9 Recording Synthesis Results

24.10 Recording Timing Results

24.11 Recording Performance Results

24.12 Creating GitHub Releases

24.13 Writing a Project Report

24.13.1 Introduction

24.13.2 Background Research

24.13.3 Project Requirements

24.13.4 Processor Architecture

24.13.5 SystemVerilog Implementation

24.13.6 Verification Method

24.13.7 FPGA Implementation

24.13.8 Performance Results

24.13.9 Discussion

24.13.10 Limitations

24.13.11 Future Work

24.13.12 Conclusion

24.14 Preparing a Supervisor Presentation

24.15 Preparing a Project Demonstration

24.16 Preparing a Poster

24.17 Preparing for Technical Questions

24.18 Creating a Portfolio Page

24.19 Summary

25. Module 21: Project Safety, Maintenance and Good Practice

25.1 Learning Objectives

25.2 Why Project Maintenance Matters

25.3 FPGA Hardware Safety

25.4 Electrostatic Discharge Precautions

25.5 Safe USB and Power Handling

25.6 Source Code Backups

25.7 GitHub Backups

25.8 Managing Vivado Versions

25.9 Managing Generated Files

25.10 Maintaining a Clean Repository

25.11 Running Regression Tests

25.12 Reviewing Synthesis Warnings

25.13 Reviewing Timing Reports

25.14 Maintaining a Project Log

25.15 Recording Design Decisions

25.16 Recording Bugs and Fixes

25.17 Recording Performance Changes

25.18 Code Review Checklist

25.19 Verification Checklist

25.20 FPGA Implementation Checklist

25.21 Maintenance Cadence

25.22 Final Project Checklist

25.23 Wrap-Up

26. Full Code for Each Module

26.1 Module 1: Project Setup Files

26.2 Module 2: Basic SystemVerilog Circuits

26.3 Module 3: Instruction Set Definitions

26.4 Module 4: Arithmetic Logic Unit

26.5 Module 5: Register File

26.6 Module 6: Program Counter and Instruction Memory

26.7 Module 7: Data Memory

26.8 Module 8: Control Unit

26.9 Module 9: Single-Cycle Processor

26.10 Module 10: Processor Testbenches

26.11 Module 11: Synthesis-Ready Processor

26.12 Module 12: Timing Constraints

26.13 Module 13: FPGA Top-Level Module

26.14 Module 14: Performance Counters

26.15 Module 15: Five-Stage Pipelined Processor

26.16 Module 16: Forwarding and Hazard Detection

26.17 Module 17: Pipelined Processor Testbench

26.18 Module 18: Processor Comparison Scripts

26.19 Module 19: Advanced Extension Examples

27. Complete Processor Code

27.1 Architecture Package

27.2 Instruction Definitions

27.3 Arithmetic Logic Unit

27.4 Register File

27.5 Program Counter

27.6 Instruction Memory

27.7 Data Memory

27.8 Immediate Generator

27.9 Control Unit

27.10 Branch Unit

27.11 Single-Cycle CPU

27.12 Pipeline Registers

27.13 Forwarding Unit

27.14 Hazard Detection Unit

27.15 Pipelined CPU

27.16 FPGA Demonstration Top Module

27.17 Complete CPU Testbench

27.18 Regression Testbench

27.19 FPGA Constraints File

27.20 Sample Program Files

28. Appendices

28.1 Appendix A: Complete Instruction Set Reference

28.2 Appendix B: Instruction Encoding Reference

28.3 Appendix C: SystemVerilog Quick Reference

28.4 Appendix D: Vivado Quick Reference

28.5 Appendix E: Git Command Reference

28.6 Appendix F: GitHub Repository Structure

28.7 Appendix G: Common Vivado Errors

28.8 Appendix H: Common SystemVerilog Errors

28.9 Appendix I: CPU Troubleshooting Guide

28.10 Appendix J: Test Program Encodings

28.11 Appendix K: Performance Calculation Examples

28.12 Appendix L: FPGA Resource Tables

28.13 Appendix M: Timing Report Examples

28.14 Appendix N: Weekly Project Log Template

28.15 Appendix O: Risk Register Template

28.16 Appendix P: Requirements Table Template

28.17 Appendix Q: Verification Matrix Template

28.18 Appendix R: Results Table Template

28.19 Appendix S: Supervisor Meeting Notes Template

28.20 Appendix T: Glossary

28.21 Appendix U: References

28.22 Appendix V: Recommended Further Reading